# Previous Experimental Plan

Quest-enabled, four-condition design before the camera-first scope reduction

Course Project Working Plan

# 1. Project Positioning

Core question: when task goal, object state, and robot state are already available, how much human manipulation information is useful for dexterous control?

Task Goal → Intermediate Manipulation Intent → Dexterous PPO → 16-DoF Allegro Hand

# 2. Research Questions

RQ1: Does intermediate manipulation intent improve dexterous control beyond task and object state alone?

RQ2: Can a low-dimensional hand-synergy representation preserve useful information contained in richer hand representations?

# 3. Fixed Experimental Platform

| Component | Choice |
| --- | --- |
| Simulator | Isaac Lab 5.1 |
| Robot | 16-DoF Allegro Hand |
| Task | Isaac-Repose-Cube-Allegro-Direct-v0 / cube reorientation |
| Controller | PPO |
| Common inputs | Robot proprioception + simulator ground-truth object state + object goal |
| Action | 16-D Allegro joint action |

# 4. Human-Intent Representations

| Condition | Human information | Role |
| --- | --- | --- |
| A. Object-only | None | Autonomous privileged-state baseline |
| B. Fingertip Geometry | Palm-local fingertips | Medium-dimensional geometric intent |
| C. PCA Hand Synergy | Low-dimensional PCA latent, primary k=4 | Core proposed representation |
| D. Full Hand | Full palm-local hand pose | High-information reference |

Wrist/palm pose is recorded but is mainly used to define the palm-local coordinate frame because the current Allegro task has a fixed palm.

# 5. Data Acquisition

Development is split into two phases so that algorithm development does not depend on immediate Quest access.

Phase I: use synthetic, procedural, or public/offline hand trajectories to build the intent-conditioning pipeline.

Phase II: collect a small real dataset with Meta Quest 3 if available. The operator manipulates a physical cube to obtain natural hand configurations.

# 6. Real Cube Tracking and Real-to-Sim Alignment

The previous plan proposed Quest 3 passthrough camera + AprilTag tracking for the physical cube. Hand and cube poses would be represented in a shared Quest/world frame, then converted to object-centric relative transforms.

Key representation: hand relative to cube, e.g.  ^cube T_hand = (^world T_cube)^-1 (^world T_hand). Relative cube rotation would be transferred to the simulator rather than globally aligning the real room with Isaac Lab.

# 7. Main Experiments

| Experiment | Comparison | Purpose |
| --- | --- | --- |
| E0 | Object-only | Measure autonomous baseline and task ceiling |
| E1 | Object-only vs Fingertips | Test whether human manipulation intent adds value |
| E2 | Fingertips vs PCA-4 vs Full Hand | Test whether compression preserves useful intent |
| Optional | PCA k={2,4,8} OR tracking noise | One focused ablation only |

# 8. Evaluation

Success Rate (SR)

Sample efficiency, e.g. N80: environment steps to reach 80% success

Learning-curve AUC

Secondary: final orientation error and representation dimension

1 seed for debugging, 3 for pilot, 5 for final results

# 9. 12-Week Timeline

| Week | Learning / Engineering | Experiment |
| --- | --- | --- |
| 1 | RL: MDP, policy gradient, actor-critic | Read Allegro task and existing baseline |
| 2 | PPO, GAE, reward design | Understand 124-D observation, 16-D action, reward/reset |
| 3 | Dexterous manipulation, contact, synergy | Formal object-only baseline |
| 4 | Human intent / recent teleoperation literature | Hand preprocessing + fingertip representation |
| 5 | PCA / hand synergy | Fingertip-conditioned PPO |
| 6 | Representation learning basics | PCA-4 PPO; first 3-way comparison |
| 7 | Experimental design / statistics | Full-hand baseline; freeze setup |
| 8 | — | 3-seed main experiment on cluster |
| 9 | — | 4 methods × 5 seeds final runs |
| 10 | — | PCA dimension OR noise ablation |
| 11 | XR / real-to-sim | Quest real-data replay; optional live demo |
| 12 | — | Plots, statistics, report, presentation |

# 10. Scope Boundary

No robot-side synergy action space.

No GPT/VLA implementation; object goal is treated as a structured task goal supplied by an upstream system.

No object perception in the simulator; use ground-truth object state.

Quest is desirable but algorithm development should begin without it.